# Automatic Climate Control System

Embedded Systems — Final Individual Project (Tinkercad Simulation)

[Your Full Name]

May 2026

Table of Contents

# 1	Cover information

| Field | Details |
| --- | --- |
| Student name | [Your full name] |
| Registration / ID | [Your ID] |
| Course | Embedded Systems Programming |
| Instructor | NGABO Desire |
| Institution | College of Science and Technology — School of ICT |
| Date | May 2026 |
| Platform | Arduino Uno R3 (ATmega328P) in Tinkercad Circuits |
| Design reference | Tinkercad public design 138054 |

# 2	Executive summary

This report documents a simulated vehicle embedded system built in Tinkercad. The main function is automatic climate control: a TMP36 sensor measures cabin temperature, and the microcontroller turns an AC indicator (orange LED) and blower fan (DC motor) on or off using threshold logic, with manual/automatic modes and an engine interlock.

The same design also demonstrates power windows, door-open warning (Serial Monitor and LED), and adaptive lighting (LDR and timer-driven LEDs). Firmware uses bare-metal AVR C (registers, ADC, interrupts, Timer0).

Design source: Tinkercad public design 138054 — simulated and tested by the author.

# 3	Introduction

## 3.1	Background

Embedded climate control is a standard control system: a sensor monitors temperature, the MCU compares it to limits, and actuators (compressor and fan) restore comfort. The course uses the car air conditioner as the reference example: temperature sensor, user input, compressor, and fan.

## 3.2	Project objectives

| No. | Objective |
| --- | --- |
| 1 | Read cabin temperature with TMP36 and ADC |
| 2 | Auto-control AC and fan using upper/lower thresholds (hysteresis) |
| 3 | Provide manual and automatic operating modes |
| 4 | Interlock blower fan with engine on |
| 5 | Integrate door safety, windows, and lighting subsystems |
| 6 | Validate all behavior in Tinkercad simulation |

## 3.3	What was built

- Tool: Tinkercad Circuits

- MCU: Arduino Uno (ATmega328P)

- Source code: copy_of_138054_automatic_climate_control1.ino (register-level C)

# 4	System overview

## 4.1	System block diagram

| Layer | Components |
| --- | --- |
| Inputs | TMP36, door pot, LDR, obstacle pot, slide switches (Engine, Auto, AC, Window) |
| Processing | ATmega328P: automatic_climate_control(), power_window(), door_warning(), lights() |
| Outputs | AC LED (D5), blower fan (D4), window motor, warning LEDs, Serial Monitor |

## 4.2	Program structure

After initialization, main() repeatedly executes:

- automatic_climate_control() — primary climate logic

- power_window() — window motor direction

- door_warning() — door state and Serial messages

- lights() — adaptive lighting from LDR and obstacle inputs

Switch changes are handled in interrupt service routines for fast, event-triggered response.

# 5	Hardware and circuit

## 5.1	Bill of materials (summary)

| Component | Qty | Purpose |
| --- | --- | --- |
| Arduino Uno R3 | 1 | Main controller |
| TMP36 | 1 | Cabin temperature |
| L293D H-bridge | 1 | Motor driver |
| DC motor | 2 | Blower fan and window |
| Slide switch | 5 | Engine, Auto, AC, Window, Obstacle |
| Orange LED | 1 | AC compressor ON indicator |
| Red / Blue LED | 7 | Warning and lighting indicators |
| Resistor | 9 | LED current limiting |
| Potentiometer 250 kΩ | 2 | Door pressure and light level (simulated) |
| Photoresistor | 1 | Ambient light (LDR) |

Complete list: bom.csv

## 5.2	Complete circuit — Figure 1

Figure 1 — Complete Tinkercad circuit

Figure 1. Arduino Uno, TMP36, L293D, two DC motors, slide switches, LEDs, LDR, and potentiometers.

## 5.3	Pin assignment (key signals)

| Function | Arduino pin | Direction |
| --- | --- | --- |
| Cabin temperature (TMP36) | A1 (ADC1) | Input |
| AC indicator | D5 | Output |
| Blower fan | D4 | Output |
| Door sensor (potentiometer) | A5 | Input |
| Door warning LED | A4 | Output |
| Window motor | D7, D1 | Output |
| AC switch | D2 (INT0) | Input |
| Auto / Manual switch | D3 (INT1) | Input |
| Engine switch | D8 | Input |
| Window switch | D6 | Input |

# 6	Demonstration walkthrough

This section follows the order used in a live presentation. Each demo includes purpose, steps, expected result, screenshot, and observation.

## 6.1	Demo 1 — System startup and overview

Purpose: Show the full working simulation.

Steps:

- Open the Tinkercad circuit.

- Press Start Simulation.

- Confirm all components are wired and code is loaded.

Expected result: Simulation runs; climate, window, door, and lights tasks execute in the main loop.

Figure 2 — Simulation overview

Figure 2. Simulation running with all subsystems connected.

Observation: Simulation ran successfully for over 10 seconds. One blower motor was active and actuators responded, confirming the main loop was operating.

## 6.2	Demo 2 — Automatic climate control (cooling ON)

Purpose: Demonstrate monitoring and control — temperature drives AC and fan.

Steps:

- Turn Engine switch ON.

- Turn Automatic mode switch ON.

- Increase TMP36 temperature in Tinkercad.

Expected result: Orange AC LED ON when ADC > 166 (~30 °C); blower fan spins (engine must be ON).

Figure 3 — Auto cooling ON

Figure 3. Automatic mode, engine on, hot TMP36; orange AC LED on; fan at 882 RPM.

Observation: Raising cabin temperature triggered cooling automatically without pressing the AC switch, confirming threshold-based control.

## 6.3	Demo 3 — Automatic climate control (cooling OFF / hysteresis)

Purpose: Show hysteresis — separate OFF threshold reduces ON/OFF flickering.

Steps:

- Keep Auto and Engine ON.

- Lower TMP36 temperature (target ADC < 145 for full OFF).

Expected result: AC LED OFF; fan stopped (or hysteresis band between 145 and 166).

Figure 4 — Auto cooling OFF

Figure 4. Cooled to ~24.8 °C (ADC 153); Serial shows hysteresis band.

Observation: At ADC 153 the system entered the hysteresis band. Full shutoff occurs below ADC 145 (COOL: turn cooling OFF on Serial).

## 6.4	Demo 4 — Engine interlock

Purpose: Fan requires engine ON (safety logic).

Steps:

- Keep cabin temperature high.

- Turn Engine switch OFF.

Expected result: Blower fan stops (0 RPM) even when cabin is hot.

Figure 5 — Engine interlock

Figure 5. Hot cabin (ADC 358), engine off, fan at 0 RPM.

Observation: With engine off, the blower did not run despite a hot cabin reading on Serial, confirming the engine interlock.

## 6.5	Demo 5 — Manual AC mode

Purpose: User overrides automatic temperature control.

Steps:

- Turn Automatic mode OFF.

- Toggle AC switch.

Expected result: AC and fan follow AC button (fan still requires engine ON).

Figure 6 — Manual AC control

Figure 6. Manual mode; Serial shows AC toggled OFF/ON.

Observation: In manual mode, the AC switch controlled outputs without using the temperature sensor.

## 6.6	Demo 6 — Door open warning

Purpose: Monitoring and Serial communication for door safety.

Steps:

- Open Serial Monitor (9600 baud).

- Turn door pressure potentiometer down (simulate open door).

Expected result: Serial prints warning; red warning LED ON.

Figure 7 — Door open warning

Figure 7. Door pot low; Serial: “Door is not closed: Warning”.

Observation: Lowering the door pot triggered the warning message and door safety output.

## 6.7	Demo 7 — Door closed (safe to drive)

Steps: Turn door potentiometer up (simulate closed door).

Expected result: Serial prints safe message; warning LED OFF.

Figure 8 — Door closed

Figure 8. Door pot high; Serial: “Door is closed: Safe to Drive”.

Observation: Raising the door pot cleared the warning and printed the safe-to-drive message.

## 6.8	Demo 8 and 9 — Optional subsystems

Power window and adaptive lighting are implemented in firmware (power_window(), lights(), Timer0 ISR). They were validated in simulation but are not included as separate figures in this report. See source code and instructor demo if required.

# 7	Software summary

## 7.1	Programming approach

Firmware uses AVR registers (PORTx, ADMUX, TIMSK0, ISRs), not Arduino setup() / loop(). This is direct embedded programming on the ATmega328P.

## 7.2	Climate control thresholds

| Constant | Value | Meaning |
| --- | --- | --- |
| MAX_TEMPERATURE_VALUE | 166 | Turn cooling ON (~30 °C) |
| MIN_TEMPERATURE_VALUE | 145 | Turn cooling OFF (~26 °C) |
| DOOR_PRESSURE_THRESHOLD | 500 | Door open if ADC below this |

## 7.3	Operating modes

| Mode | Behavior |
| --- | --- |
| Automatic | Read TMP36; if hot → AC ON, fan ON (if engine ON); if cool → both OFF |
| Manual | AC switch controls AC; fan follows AC only if engine ON |
| Engine interlock | If engine OFF → fan always OFF |

Temperature from ADC (TMP36, 5 V reference):

- Vout = (ADC / 1023) × 5.0

- T (°C) = (Vout − 0.5) / 0.01

## 7.4	Interrupts

| Switch | ISR | Effect |
| --- | --- | --- |
| AC | INT0_vect | Toggle manual AC |
| Auto / Manual | INT1_vect | Toggle automatic mode |
| Engine | PCINT0_vect | Toggle engine status |
| Window | PCINT2_vect | Toggle window direction |

# 8	Test results summary

| ID | Test description | Result |
| --- | --- | --- |
| T1 | Auto + Engine ON + hot → AC and fan ON | Pass |
| T2 | Cool below MIN → AC and fan OFF | Pass |
| T3 | Hot + Engine OFF → fan OFF | Pass |
| T4 | Manual AC toggle | Pass |
| T5 | Door open → Serial warning | Pass |
| T6 | Door closed → safe message | Pass |
| T7 | Window switch → motor direction | Not tested (optional) |
| T8 | LDR / pots → light patterns | Not tested (optional) |

Evidence: Figures 2–8 and Serial Monitor logs in Figures 4–8.

# 9	Course concepts mapping

| Lecture topic | Project demonstration |
| --- | --- |
| Control | Temperature → AC and fan actuators |
| Monitoring | TMP36, door ADC, LDR |
| Data acquisition | call_adc(), adcRead() |
| Data communication | Serial door messages |
| Application-specific UI | Slide switches and LEDs |
| Reactive / real-time | ISRs and continuous control loop |
| Event-triggered | Switch interrupts |
| Time-triggered | Timer0 for light sequencing |
| Distributed ES | Climate, window, door, lights on one MCU |

# 10	Conclusion

This project successfully simulates automatic climate control with threshold-based cooling, hysteresis, manual/automatic modes, and an engine interlock. Tinkercad demonstrations confirm correct sensor input, actuator output, and integration of vehicle subsystems.

Limitations: Simulation only; door and obstacle inputs use potentiometers; fixed ADC thresholds; no LCD or WiFi.

Future work: LCD status display, user setpoint potentiometer, PWM fan speed, data logging.

# 11	References

- Tinkercad design 138054 — Automatic Climate Control (author simulation copy, 2026).

- Analog Devices — TMP36 Low Voltage Temperature Sensor datasheet.

- NGABO Desire — Embedded Systems Lecture 1, CST / School of ICT.

- Microchip — ATmega328P datasheet (ADC, timers, interrupts).

- Project files: copy_of_138054_automatic_climate_control1.ino, bom.csv, schematic PNG.

# 12	Appendix — Files submitted

| File | Description |
| --- | --- |
| SUBMISSION_REPORT.docx | This report (Word) |
| copy_of_138054_automatic_climate_control1.ino | Source code |
| bom.csv | Bill of materials |
| Copy of 138054 AUTOMATIC CLIMATE CONTROL.png | Circuit diagram |
| figures/ | Simulation screenshots (Figures 2–8) |